configure terminal

!

! Part 1 - Cấu hình ban đầu

hostname CSW1

enable algorithm-type scrypt secret jeremysitlab

username cisco algorithm-type scrypt secret ccna

!

line console 0

login local

exec-timeout 30 0

logging synchronous

exit

!

! Part 3 - Địa chỉ IP, EtherChannel Tầng 3, HSRP

ip routing

!

interface range G1/0/2-3

no switchport

channel-group 1 mode desirable

exit

!

interface Port-Channel1

ip address 10.0.0.41 255.255.255.252

exit

!

interface G1/0/1

no switchport

ip address 10.0.0.34 255.255.255.252

exit

!

interface G1/1/1

no switchport

ip address 10.0.0.45 255.255.255.252

exit

!

interface G1/1/2

no switchport

ip address 10.0.0.49 255.255.255.252

exit

!

interface G1/1/3

no switchport

ip address 10.0.0.53 255.255.255.252

exit

!

interface G1/1/4

no switchport

ip address 10.0.0.57 255.255.255.252

exit

!

interface Loopback0

ip address 10.0.0.77 255.255.255.255

exit

!

interface range G1/0/4-24

shutdown

exit

!

! Part 4 - Giao thức Rapid Spanning Tree (Rapid PVST+)

spanning-tree mode rapid-pvst

!

! Part 5 - Định tuyến tĩnh và định tuyến động (OSPF)

router ospf 1

router-id 10.0.0.77

passive-interface Loopback0

network 10.0.0.41 0.0.0.0 area 0

network 10.0.0.34 0.0.0.0 area 0

network 10.0.0.45 0.0.0.0 area 0

network 10.0.0.49 0.0.0.0 area 0

network 10.0.0.53 0.0.0.0 area 0

network 10.0.0.57 0.0.0.0 area 0

network 10.0.0.77 0.0.0.0 area 0

exit

!

interface range G1/0/1,G1/1/1-4

ip ospf network point-to-point

exit

!

! Part 6 - Dịch vụ mạng: DHCP, DNS, NTP, SNMP, Syslog, SSH, CDP/LLDP

ip domain name jeremysitlab.com

ip name-server 10.5.0.4

!

ntp authentication-key 1 md5 ccna

ntp trusted-key 1

ntp server 10.0.0.76 key 1

!

snmp-server community SNMPSTRING RO

!

logging 10.5.0.4

logging trap debugging

logging buffered 8192

!

crypto key generate rsa modulus 4096

! **LƯU Ý:** Lệnh trên là tương tác. Sau khi dán, bạn cần **nhập `4096` và nhấn Enter** khi được nhắc để tạo khóa RSA. Sau đó, chạy tiếp các lệnh còn lại.

!

ip ssh version 2

access-list 1 permit 10.1.0.0 0.0.0.255

!

line vty 0 15

access-class 1 in

transport input ssh

login local

logging synchronous

exit

!

no cdp run

lldp run

!

exit

! **LƯU Ý:** Lệnh `exit` cuối cùng này sẽ đưa bạn ra khỏi chế độ cấu hình toàn cục.

!

do write