configure terminal

!

! Part 1 - Cấu hình ban đầu

hostname DSW-A1

enable algorithm-type scrypt secret jeremysitlab

username cisco algorithm-type scrypt secret ccna

!

line console 0

login local

exec-timeout 30 0

logging synchronous

exit

!

! Part 2 - VLANs và Layer-2 EtherChannel

! EtherChannel với DSW-A2 (PAgP)

interface range G1/0/4-5

channel-group 1 mode desirable

exit

!

! Cấu hình Trunk đến Access Switches

interface range G1/0/1-3

switchport mode trunk

switchport nonegotiate

switchport trunk native vlan 1000

switchport trunk allowed vlan 10,20,40,99

exit

!

! Cấu hình Trunk trên Port-Channel1

interface Port-Channel1

switchport mode trunk

switchport nonegotiate

switchport trunk native vlan 1000

switchport trunk allowed vlan 10,20,40,99

exit

!

! VTP Server

vtp domain JeremysITLab

vtp version 2

!

! Tạo VLANs (DSW-A1 là VTP server cho Office A)

vlan 10

name PCs

exit

vlan 20

name Phones

exit

vlan 40

name Wi-Fi

exit

vlan 99

name Management

exit

!

! Tắt các cổng không sử dụng

interface range G1/0/6-24,G1/1/3-4

shutdown

exit

!

! Part 3 - Địa chỉ IP, EtherChannel Tầng 3, HSRP

ip routing

!

! Cấu hình địa chỉ IP trên các giao diện vật lý và loopback

interface G1/1/1

no switchport

ip address 10.0.0.46 255.255.255.252

exit

!

interface G1/1/2

no switchport

ip address 10.0.0.62 255.255.255.252

exit

!

interface Loopback0

ip address 10.0.0.79 255.255.255.255

exit

!

! Cấu hình HSRP

! VLAN 99 (Management) - DSW-A1 Active

interface Vlan99

ip address 10.0.0.2 255.255.255.240

standby version 2

standby 1 ip 10.0.0.1

standby 1 priority 105

standby 1 preempt

exit

!

! VLAN 10 (PCs) - DSW-A1 Active

interface Vlan10

ip address 10.1.0.2 255.255.255.0

standby version 2

standby 2 ip 10.1.0.1

standby 2 priority 105

standby 2 preempt

exit

!

! VLAN 20 (Phones) - DSW-A1 Standby

interface Vlan20

ip address 10.2.0.2 255.255.255.0

standby version 2

standby 3 ip 10.2.0.1

exit

!

! VLAN 40 (Wi-Fi) - DSW-A1 Standby

interface Vlan40

ip address 10.6.0.2 255.255.255.0

standby version 2

standby 4 ip 10.6.0.1

exit

!

! Part 4 - Giao thức Rapid Spanning Tree (Rapid PVST+)

spanning-tree mode rapid-pvst

!

! Cấu hình mức ưu tiên STP

spanning-tree vlan 10,99 priority 0

spanning-tree vlan 20,40 priority 4096

!

! Part 5 - Định tuyến tĩnh và định tuyến động (OSPF)

router ospf 1

router-id 10.0.0.79

passive-interface Loopback0

passive-interface Vlan10

passive-interface Vlan20

passive-interface Vlan40

network 10.0.0.46 0.0.0.0 area 0

network 10.0.0.62 0.0.0.0 area 0

network 10.0.0.79 0.0.0.0 area 0

network 10.0.0.2 0.0.0.0 area 0

network 10.1.0.2 0.0.0.0 area 0

network 10.2.0.2 0.0.0.0 area 0

network 10.6.0.2 0.0.0.0 area 0

exit

!

interface range G1/1/1-2

ip ospf network point-to-point

exit

!

! Part 6 - Dịch vụ mạng: DHCP, DNS, NTP, SNMP, Syslog, SSH, CDP/LLDP

! DHCP Relay Agent

interface Vlan10

ip helper-address 10.0.0.76

exit

interface Vlan20

ip helper-address 10.0.0.76

exit

interface Vlan40

ip helper-address 10.0.0.76

exit

interface Vlan99

ip helper-address 10.0.0.76

exit

!

! DNS

ip domain name jeremysitlab.com

ip name-server 10.5.0.4

!

! NTP

ntp authentication-key 1 md5 ccna

ntp trusted-key 1

ntp server 10.0.0.76 key 1

!

! SNMP

snmp-server community SNMPSTRING RO

!

! Syslog

logging 10.5.0.4

logging trap debugging

logging buffered 8192

!

! SSH

crypto key generate rsa modulus 4096

! **LƯU Ý:** Lệnh trên là tương tác. Sau khi dán, bạn cần **nhập `4096` và nhấn Enter** khi được nhắc để tạo khóa RSA. Sau đó, chạy tiếp các lệnh còn lại.

!

ip ssh version 2

access-list 1 permit 10.1.0.0 0.0.0.255

!

line vty 0 15

access-class 1 in

transport input ssh

login local

logging synchronous

exit

!

! CDP/LLDP

no cdp run

lldp run

!

! Part 7 - Bảo mật: ACL

ip access-list extended OfficeA_to_OfficeB

permit icmp 10.1.0.0 0.0.0.255 10.3.0.0 0.0.0.255

deny ip 10.1.0.0 0.0.0.255 10.3.0.0 0.0.0.255

permit ip any any

exit

!

interface Vlan10

ip access-group OfficeA_to_OfficeB in

exit

!

exit

! **LƯU Ý:** Lệnh `exit` cuối cùng này sẽ đưa bạn ra khỏi chế độ cấu hình toàn cục.

!

do write