configure terminal

!

! Part 1 - Cấu hình ban đầu [1-10]

hostname R1

enable secret jeremysitlab

username cisco secret ccna

!

line console 0

login local

exec-timeout 30 0

logging synchronous

exit

!

! Part 3 - Địa chỉ IP, EtherChannel Tầng 3, HSRP [11-15]

interface range G0/0/0,G0/1/0

ip address dhcp

no shutdown

exit

!

interface G0/0

ip address 10.0.0.33 255.255.255.252

no shutdown

exit

!

interface G0/1

ip address 10.0.0.37 255.255.255.252

no shutdown

exit

!

interface Loopback0

ip address 10.0.0.76 255.255.255.255

exit

!

! Part 5 - Định tuyến tĩnh và định tuyến động (OSPF) [16-26]

router ospf 1

router-id 10.0.0.76

passive-interface Loopback0

exit

!

interface Loopback0

ip ospf 1 area 0

exit

!

interface range G0/0-1

ip ospf 1 area 0

ip ospf network point-to-point

exit

!

ip route 0.0.0.0 0.0.0.0 203.0.113.1

ip route 0.0.0.0 0.0.0.0 203.0.113.5 2

!

router ospf 1

default-information originate

exit

!

! Part 6 - Dịch vụ mạng: DHCP, DNS, NTP, SNMP, Syslog, FTP, SSH, NAT [26-56]

ip dhcp excluded-address 10.0.0.1 10.0.0.10

ip dhcp excluded-address 10.1.0.1 10.1.0.10

ip dhcp excluded-address 10.2.0.1 10.2.0.10

ip dhcp excluded-address 10.0.0.17 10.0.0.26

ip dhcp excluded-address 10.3.0.1 10.3.0.10

ip dhcp excluded-address 10.4.0.1 10.4.0.10

ip dhcp excluded-address 10.6.0.1 10.6.0.10

!

ip dhcp pool A-Mgmt

network 10.0.0.0 255.255.255.240

default-router 10.0.0.1

domain-name jeremysitlab.com

dns-server 10.5.0.4

option 43 ip 10.0.0.7

exit

!

ip dhcp pool A-PC

network 10.1.0.0 255.255.255.0

default-router 10.1.0.1

domain-name jeremysitlab.com

dns-server 10.5.0.4

exit

!

ip dhcp pool A-Phone

network 10.2.0.0 255.255.255.0

default-router 10.2.0.1

domain-name jeremysitlab.com

dns-server 10.5.0.4

exit

!

ip dhcp pool B-Mgmt

network 10.0.0.16 255.255.255.240

default-router 10.0.0.17

domain-name jeremysitlab.com

dns-server 10.5.0.4

option 43 ip 10.0.0.7

exit

!

ip dhcp pool B-PC

network 10.3.0.0 255.255.255.0

default-router 10.3.0.1

domain-name jeremysitlab.com

dns-server 10.5.0.4

exit

!

ip dhcp pool B-Phone

network 10.4.0.0 255.255.255.0

default-router 10.4.0.1

domain-name jeremysitlab.com

dns-server 10.5.0.4

exit

!

ip dhcp pool Wi-Fi

network 10.6.0.0 255.255.255.0

default-router 10.6.0.1

domain-name jeremysitlab.com

dns-server 10.5.0.4

exit

!

ip domain name jeremysitlab.com

ip name-server 10.5.0.4

!

ntp master 5

ntp server 216.239.35.0

ntp authentication-key 1 md5 ccna

ntp trusted-key 1

!

snmp-server community SNMPSTRING RO

!

logging 10.5.0.4

logging trap debugging

logging buffered 8192

!

ip ftp username cisco

ip ftp password cisco

!

crypto key generate rsa modulus 4096

! **LƯU Ý:** Lệnh trên là tương tác. Sau khi dán, bạn cần **nhập `4096` và nhấn Enter** khi được nhắc để tạo khóa RSA. Sau đó, chạy tiếp các lệnh còn lại.

!

ip ssh version 2

access-list 1 permit 10.1.0.0 0.0.0.255

!

line vty 0 15

access-class 1 in

transport input ssh

login local

logging synchronous

exit

!

ip nat inside source static 10.5.0.4 203.0.113.113

!

interface range G0/0/0,G0/1/0

ip nat outside

exit

!

interface range G0/0-1

ip nat inside

exit

!

access-list 2 permit 10.1.0.0 0.0.0.255

access-list 2 permit 10.2.0.0 0.0.0.255

access-list 2 permit 10.3.0.0 0.0.0.255

access-list 2 permit 10.4.0.0 0.0.0.255

access-list 2 permit 10.6.0.0 0.0.0.255

!

ip nat pool POOL1 203.0.113.200 203.0.113.207 netmask 255.255.255.248

ip nat inside source list 2 pool POOL1 overload

!

no cdp run

lldp run

!

exit

! **LƯU Ý:** Lệnh `exit` cuối cùng này sẽ đưa bạn ra khỏi chế độ cấu hình toàn cục.

!

do write